Project

Low Dropout Regulator

By: Amar Bharat Khatri

- Introduction

The objective of this project was to design a fully integrated Low Dropout (LDO) voltage regulator operating from a 1.8 V input to generate a regulated 1.6 V output for load currents ranging from 20 mA to 100 mA. The design targeted high power efficiency (≥ 91.1%), PSRR of around −40 dB at 100 kHz, low output voltage error, and tight line and load regulation. To ensure on-chip integration, the output and compensation capacitors were constrained to 500 pF and 15 pF, respectively. The regulator was also designed to achieve fast transient response with minimal voltage overshoot/undershoot and stable operation across the specified temperature range.

- Circuit Diagram

Fig 1: LDO Schematic

| Parameters | Values |
| --- | --- |
| Vin | 1.8V |
| Load Capacitance | 500p F |
| Vref | 1V |
| R1 | 60K |
| R2 | 100K |
| Load current range | 20mA to 100mA |
| Nominal Load | 50mA |
| Miller Capacitor | 8p |

Table1 : Parameters

Calculations:

- Transistor Sizing:

| Transistor | Width (W) (um) | Length (L) (um) | Multiplier | W/L | Vov (mV) (Nominal) |
| --- | --- | --- | --- | --- | --- |
| M3 &M4 (Pmos current mirror) | 85 | 1 | 1 | 85/1 | 186 |
| M0 & M2 (Nmos diff Pair) | 45 | 1 | 1 | 45/1 | 95 |
| M1 (Tail of OTA) | 75 | 5 | 1 | 75/1 | 190 |
| M5 (Pass Transistor) | 100 | 0.18 | 200 | 20000/0.18 | 101 |

Table 2 : Sizings

- Stability Table:

| Load (mA) | Phase Margin | BandWidth | Loop Gain | Gain Margin |
| --- | --- | --- | --- | --- |
| 20 | 62.865 degree | 7.49M | 69.78 dB | 16.474 db |
| 50 | 68.32 degree | 7.29M | 67.57 dB | 19.908 dB |
| 100 | 71.269 degree | 6.69M | 63.25 dB | 22.883 dB |

Table 3: Stability

- Power Consumption Table:

| Blocks | Light Load | Nominal Load | Full Load |
| --- | --- | --- | --- |
| OTA | 282.6 uW | 282 uW | 284 uW |
| Feedback resistors | 16 uW | 16 uW | 16 uW |
| Pass Transistor | 4 mW | 10 mW | 20 mW |

Table 4: Power

- Line regulation: Target was to achieve < 1% variation for variation of 2% of Line Voltage above and below Vout = 1.6.

Achieved..

Plot : Line regulation.

- Load Regulation: Target was to achieve < 2% variation from light load to full load.

Achieved..

Plot : Load Regulation

- Power Supply Rejection Ratio : Target @100Hz approx -40db

Achieved..

Plot : PSRR (Vdd)

- Transient Response : Target ≤ 150 mV overshoot/undershoot for a 20 mA to 50 mA load step (both up and down with edge time of 10 ns) with settling time (output voltage within ±2% of the regulated value) less than 150 ns.

Achieved..

Plot : Transient response with overshoot

Plot : Transient response with undershoot.

- Loop Stability graphs:

- I_load = 20mA (Light Load)

- I_load = 50mA (Nominal Load)

- I_load = 100mA (Full Load)

- Final Summary Table:

| Parameters | Results |
| --- | --- |
| PSRR | -38.347 @ 100KHz |
| Load Regulation ( ΔVout/ΔLoad *100) | <1.1% across load range |
| Line Regulation | <0.862% for ± 2% of Input Voltage |
| Phase Margin | > 63° across load range |
| Efficiency | > 88.45% across Load Range |
| DC error | 0.09mV at Nominal Load and <57mV across Load Range |
| Transient response | Settling Time < 155ns |
|  | Overshoot and Undershoot < 120mV |

Table 5: Summary